# PFCS setup instructions for PCM.

- Launch “ICS Connect” from programs menu or double click the shortcut on the desktop.

- Select QX series from Family dropdown.

# General Setup

- Click on Setup -> IC-PCM 1:1 General Setup.

# Controller Selection

- Select the Controller form controller ID dropdown List.

# Enable PFCS

- Click on “PFCS Settings” to set the PFCS settings.

- Click on “Enable PFCS”

- Select “Ethernet” under Type.

- Click on “Enable Unsolicited Port”.

- Enter IP address for the PFD server.

- Enter solicited port.

- Enter unsolicited port.

- Enter solicited Machine ID.

- Enter Unsolicited Machine ID.

# Map Configuration and Machine IDs

- Click on “Config Machine Ids” to enter the Machine ID for each PSet/configs.

- Click on “Config Machine IDs” button to set Machine IDs for each PSet/configs.

- Once all the settings are in place click on “Send” button.

# Barcode Setup

- When Enable Unsolicited Port is checked, Barcode is sent as unsolicited message to select a specific config.

- In order to setup the barcode, select the Barcode Settings Tab

- Barcode can be set to OFF, Active or Passive.  Set to Active.

- Set the Length of the barcode as 8.

- Set Criteria setting, 1 in “From” and 8 in “To”.

- Double click on the Grid.

- Click on “Add” button to add the config Criteria.

- Repeat the steps for configuration 2 from the dropdown box window.

- Once the settings are complete, click on “OK” and the settings will be reflected as below.

- Click on Send button and reboot controller after 30 sec in order to reflect the changes.

# Configuration Selection type

- In order to use PFCS on ICDM controller, Configuration select should be set to “External Ethernet”. This setting can be found under “Setup->General” and “Spindle Initialization” tab.

- Click on Send button.