WU-1

(Battery system wrench wireless unit)

Communication specifications

(The 1st edition).

August 31, 2015

Yokota industrial Co.,Ltd

- Specification of serial output (RS-232C)

Here is the serial communication (RS232C) specifications.

|  | Back connector |
| --- | --- |
| Transmission scheme | Asynchronous communication method |
| Connection | DTE |
| Transmission speed | To be selected by setting (2400bps,4800bps,9600bps, 19200bps, 38400bps, 115200bps) |
| Character configuration | Start bit: 1bit. Data length: 8bit. Stop bit: 1bit. To be selected by setting Parity :(None,odd parity,even parity) |
| Character Code | ASCII(Alphanumeric character and control code) |
| Connector on the controller side | D-Sub 9P male |

Table 1.1. Specification of serial communication (RS232C)

(Note) Those setting items underlined with bold face are the default values.

Use the cross (reverse) cable in order to connect with the host (DTE specification) because the connector has DTE specification.

Connection to the host

| The Host (DTE specification) |  |  |  | WU-1 (DTE specification) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Contents | Signal name | Terminal No |  | Terminal No | Signal name | Contents |
| Data Carrier Detect | DCD | 1(8) |  | 1 |  |  |
| Received data | RxD | 2(3) |  | 2 | RxD | Received data |
| Transmitted Data | TxD | 3(2) |  | 3 | TxD | Transmitted Data |
| Terminal Ready | DTR | 4(20) |  | 4 | DTR | Terminal Ready |
| Ground | GND | 5(7) |  | 5 | GND | Ground |
| Data set ready | DSR | 6 |  | 6 | DSR | Data set ready |
| Request To Send | RTS | 7(4) |  | 7 | RTS | Request To Send |
| Clear To Send | CTS | 8(5) |  | 8 | CTS | Clear To Send |
| Ring Indicator | RI | 9(22) |  | 9 |  |  |

The terminal number of the host indicates D-Sub 9 pin. The number of inside ( ) indicates D-sub 25 pin.

2. LAN (Ethernet)

## 2.1. Ethernet specification.

|  |  | Initial value |
| --- | --- | --- |
| Connector shape | RJ-45 |  |
| Interface | 100BASE-TX/10BASE-T (Automatic recognition) |  |
| Protocol | TCP/IP (Server function) |  |
| IP address | Arbitrarily setting, input range: 0～255.0～255.0～255.0～255 | 192.168.124.20 |
| Subnet Mask | Arbitrarily setting, input range: 0～255.0～255.0～255.0～255 | 255.255.255.0 |
| Port number | Arbitrarily setting, input range: 0～65535 | 10001 |
| ID number (controller identification number) | Arbitrarily setting, input range: 0～9999 | 0 |

Taben2.1. Ethernet specification

## 2.2. LAN communication starting procedure

It is necessary to open the line from the host to WU-1 to start the communication.

ACK

SYN/ACK

WU-1

Host

Fig. 2.2 Time flow chart to start LAN communication.

- Contents of communication

3.1. Basic format.

Send CR and LF as a separator followed after the cable. Transmission is only once and it will not be resent.

## 3.2. When Bolt Number Management Function ON.

| ID number | Tool Number | Work name | Work serial number | - | Remain number | Total judgment | Output Torque value | Torque judgment | Free running Angle |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 《4》 | (1) | 【1】 | 【4】 | (1) | (2) | 【1】 | 【5】 | 【1】 | 【5】 |  |

|  | Tightening angle | Free running Time | Tightening, Time. | Number of pulses | Angle judgment | Blank | Time |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 【4】 | 【5】 | 【5】 | (4) | 【3】 | (1) | (15) |

## 3.3. At bolt number management OFF

| ID number | Tool Number | Work name | Total tightening quantity | Torque judgment | Output torque value | Torque Judgment | Free running Angle |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 《4》 | (1) | 【1】 | 【4】 | 【1】 | 【5】 | 【1】 | 【5】 |  |

|  | Tightening angle | Free running Time | Tightening, Time. | Number of tightening pulses | Angle judgment | Blank | Time |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 【4】 | 【5】 | 【5】 | (4) | 【3】 | (1) | (15) |

The value in 【 】, ( ), and 《 》 of the column of each item at the bottom is the number of characters (number of byte). 【 】is output without fail and ( ) and 《 》 are output when output designation is on.

When the output designation of ( ) is off, in case of variable length data, the number of characters inside ( ) is output without a gap and output with space (blank) in case of fixed length data.

Regardless of valiable or fixed length data, when the output designation is off, the items are output without a gap.

Tool ID is output by LAN communication only.  It is output when the ID output designation is ON.

3.4. Example of output format

|  |  | Byte number | Group management ON |  | Group management OFF |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Variable-length | Fixed length | Variable-length | Fixed length |
| 1 | ID number | 4 | ×S | ×S | ×S | ×S |
| 2 | Tool number | 1 | ×S | △S | ×S | △S |
| 3 | Work name | 1 | ◯ | ◯ | ◯ | ◯ |
| 4 | Work serial number | 4 | ◯ | ◯ |  |  |
| 5 | ‘―' | 1 | ×S | △S |  |  |
| 6 | Remain number | 2 | ×S | △S |  |  |
| 7 | Total Tightened bolt | 4 |  |  | ◯ | ◯ |
| 8 | Total judgment | 1 | ◯ | ◯ | ◯ | ◯ |
| 9 | Output torque value | 5 | ◯ | ◯ | ◯ | ◯ |
| 10 | Torque judgment | 1 | ◯ | ◯ | ◯ | ◯ |
| 11 | Free running angle | 5 | ◯ | ◯ | ◯ | ◯ |
| 12 | Tightening angle | 4 | ◯ | ◯ | ◯ | ◯ |
| 13 | Free running time | 5 | ◯ | ◯ | ◯ | ◯ |
| 14 | Tightening, Time. | 5 | ◯ | ◯ | ◯ | ◯ |
| 15 | Number of tightening pulses | 4 | ×S | △S | ×S | △S |
| 16 | Angle judgment | 3 | ◯ | ◯ | ◯ | ◯ |
| 17 | Blank | 1 | ×△ | ×△ | ×△ | ×△ |
| 18 | Time | 15 | ×S | ×S | ×S | ×S |
|  | Total Byte number |  | 34 ¦ 62 | 42 ¦ 62 | 34 ¦ 59 | 39 ¦ 59 |

×S: Output without a gap when no output according to the setting.

△S: Space when no ouput according to the setting.

×△: Output without a gap when no output of time.

Table 3.1 Details of text contents

| Item | Reference |
| --- | --- |
| Tool ID | Indicates preset tool ID |
| Tool number | Output the tool number used to tighten. 1 digit of 1~4 |
| Work Name | Output tightened work name. 1 character from a to d (small letter) |
| Batch serial number | Serial number of the tightened batch. 4 digit of △△△1~9999 |
| Remain number | When the number management is ON, output remain number of the corresponding work number. 2 digit of △0~99 |
| Total tightening number | When the number management is OFF, ouput the total tightening number. 2 digit of △0~99 |
| Total judgemnt | Output the judgment concerning the tightening.(A:pass、N:failed) （Total judgment is a combination with the judgment like the torque judgment and the angle etc.） Result of angle etc.. Torque result; OK; NG. Aok; n31～n37. Torque OK; K; A; N. Over; V; N. SLOW2; s; A; N. UNDER; U; N. SLOW; S; N. |
| Output torque value | Output the working torque value to the output axis. |
| Torque judgment | Output the judgment result concerning the output torque value. (K:OK、V:OVER、U:UNDER、S:SLOW.、s:SLOW2) |
| Angle of free running | Output the angle where the output axis will rotate during free running. 4 digit of △△△△0~9999 |
| Tighening angle | Output the angle where the output axix of the tool will rotate by the time it reaches the tightening torque from the display start torque. 3 digit of △△△０～９９９ |
| Free running time | Output time used for free running. 4 digit of △△△0~9999 （unit：sec） |
| Tighetneing time | Output time from the display start torque to tightening completion. 4 digits of △△△0~9999（unit：sec） |
| Number of tightening pulses | Output number of tightening pulses from the display start torque to tightening completion. 4 digits of △△△1~△△51 |
| Angle judgment | Aok: OK, n31: Free runnig angle is less than the lower limit. n32: Free running angle is over the upper limit. n35: The tightening angle is less than lower limit. n37:The tightening angle is over the uppler limit. |
| Time | Output the time of tightening opereation Month/Day time: min: sec 15 digit of △01/01△00:00:00~△12/31△23:59:59” |