# PFCS setup instructions for ICDM.

- Launch “ICS Connect” from programs menu or double click the shortcut on the desktop.

# General Setup

- Click on Setup -> General.

# Controller Selection

- Select the Controller from controller ID dropdown List.

# Enable PFCS

- Click on “Comm Protocol Assignment” tab to set the PFCS settings.

- Select the 3rd dropdown box and select “Single Spindle PFCS”

- Make the necessary setting under PFCS settings such as:

- KeepAlive Timeout – Time in seconds for every Keepalive message to send.

- Retry Count – Number of time the Retry should happen.

- Retry Timeout – Time in seconds for the next retry.

- IP Address – PFCS server IP address

- Solicited Port – Port settings for Solicited communication.

- Unsolicited Port – Port settings for unsolicited communication.

- Solicited MachineID – Machine ID for Solicited port

- Unsolicited Machine ID – Machine ID for Unsolicited port.

- Enable/Disable Unsolicited Port – Check the box to enable the Unsolicited Port.

- Config Machine IDs – Machine IDs for the corresponding configs/ pset.

# Map Configuration and Machine IDs

- Click on “Config Machine IDs” button to set Machine IDs for each PSet/configs.

- Once all the settings are in place click on “Send” button.

# Barcode Setup

- When Unsolicited Port is enabled, Barcode can be sent as unsolicited message to select the config.

- In order to setup the barcode, select the second dropdown box and select Barcode/Vin

- Barcode can be set to OFF, Active or Passive. Set it to Active.

- Set the Length of the barcode as 8.

- Set Criteria setting, 1 in “From” and 8 in “To”.

- Double click on the Grid.

- Click on “Add” button to add the config Criteria.

- Repeat the steps for configuration 2 from the dropdown box window.

- Once the settings are complete, click on “OK” and the settings will be reflected as below.

- Click on Send button and reboot controller after 30 sec in order to reflect the changes.

# Configuration Selection type

- In order to use PFCS on ICDM controller, Configuration select should be set to “External Ethernet”. This setting can be found under “Setup->General” and “Spindle Initialization” tab.

- Click on Send button.

Controller is ready for PFCS protocol communication.